Check Engine Code Instructions

Directions for checking the Codes

1)  Remove the kick panel just beside the left foot rest on the driver's side. 
   2)  Locate blue wires. 
   3)  Locate black connector. Hold it notch up. 
   4)  With ignition off, insert one of the wires into top row, 2nd pin from left. 
   5)  Turn key to on position, but do not start the car. 
   6)  Watch blinky check engine light on dashboard. Count blinks. Long blinks = 10, Short blinks = 1. 
   7)  Keep watching blinky light until you get mesmorized.

To clear codes (after problem has been resolved): 
   1)  With engine off, stick the blue wires to 1st and 2nd pin in the top row counting from left on the

black connector, with notch facing up. 
   2)  Start car. Shut off car. Remove pins. 
   3)  Read codes to make sure they are clear. No codes = steady blinks at 1/4 second intervals.

CODE PROBABLE CAUSE 
  11  Crank Angle Sensor or Circuit 
  12  Starter Switch or Circuit 
  13  Cam Position Sensor or Circuit (TDC Sensor on Justy) 
  14  Fuel Injector No. 1 (Legacy, Impreza, Justy, SVX) 
  14  Fuel Injector No. 1 and 2 (XT, Loyale, GL, DL) 
  15  Fuel Injector No. 2 (Legacy, Impreza, Justy, SVX) 
  15  Fuel Injector No. 3 and 4 (Loyale, GL, DL) 
  15  Fuel Injector No. 5 and 6 (XT-6) 
  16  Fuel Injector No. 3 (Legacy, Impreza, Justy, SVX) 
  16  Fuel Injector No. 3 and 4 (XT) 
  17  Fuel Injector No. 4 (Legacy, Impreza, SVX) 
  17  Fuel injector No. 1 and 2 (XT-6) 
  18  Fuel Injector No. 5 (SVX) 
  19  Fuel Injector No. 6 (SVX) 
  21  Coolant Temperature Sensor or Circuit 
  22  Knock Sensor or Circuit (Right Side on SVX) 
  23  Air Flow Meter or Circuit (Exc. Justy) 
  23  Pressure Sensor (Justy) 
  24  Air Control Valve or Circuit (Exc. Justy) 
  24  Idle Speed Control Solenoid Valve (Justy) 
  25  Fuel injector No. 3 and 4 (XT-6) 
  26  Air Temperature Sensor (Justy) 
  28  Knock Sensor No. 2 (SVX, Left Side) 
  29  Crank Angle Sensor (SVX, No. 2) 
  31  Throttle Position Sensor or Circuit 
  32  Oxygen Sensor or Circuit (No. 1, Right Side, On SVX) 
  33  Vehicle Speed Sensor (VSS) or Circuit 
  34  EGR Solenoid or Circuit 
  35  Purge Control Solenoid or Circuit 
  36  Air Suction Solenoid Valve (Impreza) 
  36  Ignitor Circuit (Justy) 
  37  Oxygen Sensor (No. 2, Left Side, On SVX) 
  38  Engine Torque Control (SVX) 
  41  Air/Fuel Adaptive Control 
  42  Idle Switch or Circuit 
  43  Throttle Switch (Justy) 
  44  Wastegate Duty Solenoid (Turbo Models) 
  45  Pressure Sensor Duty Solenoid (Turbo Models) 
  45  Atmospheric Pressure Sensor or Circuit (Non- Turbo Models) 49 Airflow Sensor

CODE PROBABLE CAUSE Continued:

51  Neutral Switch (Manual Transmission Models) 
  51  Inhibitor Switch (Automatic Transmission Models) 
  52  Parking Brake Switch (Exc. Justy) 
  52  Clutch Switch (Justy) 
  55  EGR Temperature Sensor or Circuit 
  56  EGR System 
  61  Parking Brake Switch (Loyale) 
  61  Fuel Tank Pressure Control Solenoid (Impreza) 
  62  Fuel Temperature Sensor (Impreza) 
  62  Electric Load Signal (Justy) 
  63  Fuel Tank Pressure Sensor (Impreza) 
  63  Blower Fan Switch (Justy) 
  65  Vacuum Pressure Sensor

To perform the TCU self diag test for EXISTING problems

1)  Warm up engine by driving at speeds greater than 12mph. 
  2)  Stop vehicle and turn ignition switch OFF. 
  3)  Turn ignition switch ON and make sure POWER indicator lamp comes on. 
  4)  Turn ignition switch OFF. 
  5)  Move selector lever to D and turn manual switch ON. 
  6)  Turn ignition switch ON. 
  7)  Move selector lever to "3" and turn manual switch OFF. 
  8)  Move selector level to "2" and turn manual switch ON. 
  9)  Move selector lever to "1" and turn manual switch OFF. 
10)  Partially depress accelerator pedal (to turn idle switch off). 
11)  Check code as displayed on POWER light. Blinking once every 1/4sec is normal.

To perform the TCU self diag test for PREVIOUS problems

1)  Warm up engine by driving at speeds greater than 12mph. 
  2)  Stop vehicle and turn ignition switch OFF. 
  3)  Turn ignition switch ON and make sure POWER indicator lamp comes on. 
  4)  Turn ignition switch OFF. 
  5)  Move selector lever to "1" and turn manual switch ON. 
  6)  Turn ignition switch ON. 
  7)  Move selector lever to "2" and turn manual switch OFF. 
  8)  Move selector lever to "3" and turn manual switch ON. 
  9)  Move selector lever to "D" and trun manual switch OFF. 
10)  Partially depress accelerator pedal (to turn idle switch off). 
11)  Check code as displayed on POWER light. Blinking once every 1/4sec is normal.

Trouble codes: 
11.  Duty solenoid A 
12.  Duty solenoid B 
13.  Shift solenoid 3 
14.  Shift solenoid 2 
15.  Shift solenoid 1 
21.  ATF temp sensor 
22.  Atmospheric sensor 
23.  Engine revolution signal 
24.  Duty solenoid C 
25.  Engine torque control signal 
31.  Throttle sensor 
32. Vehicle speed sensor 1 
33. Vehicle speed sensor 2